DWC_macsec 1.05a Driver Analysis

Architecture, Data Path, and Implementation Notes

# 1. Memory Architecture

## 1.1 SAI vs SAD Memory Type

From DWC_macsec 1.05a configuration (DWC_macsec - 1.05a.html):

| Module | Memory Type | Status |
| --- | --- | --- |
| TX/RX SAI | Internal FFs | Active |
| TX/RX SAD | External Dual Port SRAM | Active |
| TX/RX SALT | External Two Port SRAM | Inactive (XPN=0) |
| RX ARW | External Two Port SRAM | Active |
| TX/RX MIB (SA+SC) | External Two Port SRAM | Active |
| AES Key/Context | External SRAM | Active (98 contexts) |

SAI uses Internal FFs for low-latency combinational lookup. SAD/ARW/MIB use external SRAM because: (a) entry count is larger (96 SAD vs 48 SAI), (b) entry width is larger (200+ bits per SAD), (c) every packet requires PN read-modify-write in SAD and ARW window update.

## 1.2 SAI vs SAD Functional Separation

SAI (Security Association Identifier): Lookup table that maps incoming frame fields (MAC/SCI/VLAN) to an SA index. Read-dominated, configured once.

SAD (Security Association Data): Parameter storage per SA - PN counter, Key index, TCI, MTU, TTL. Read-write on every packet (PN increment).

The two-table split is analogous to TLB vs Page Table: SAI is the fast lookup cache, SAD is the backing storage.

## 1.3 TCI Fields (TX_SAD_0_HIGH)

TCI is stored in SAD, NOT SAI. It is an OUTPUT parameter written into the SecTAG of MACsec frames:

| Bit | Name | Meaning |
| --- | --- | --- |
| 4 | ES (End Station) | Source and destination are end stations |
| 3 | SC (Secure Channel) | Secure Channel is supported |
| 2 | SCB (Single Copy Broadcast) | Broadcast frame using shared SA (currently 0) |
| 1 | E (Encryption) | Payload is encrypted |
| 0 | C (Changed Text) | Integrity protection is applied |

Current default: DWC_TCI_ENCRYPT_AUTH = 0x0B (ES=0, SC=1, SCB=0, E=1, C=1). SCB=0 is correct for unicast-only MACsec operation.

# 2. PN Management and Key Rotation

## 2.1 SOFT_TTL and HARD_TTL

Two-tier early warning system for PN exhaustion in the TX SAD:

| Threshold | SAD Register | IRQ Bit | Behavior |
| --- | --- | --- | --- |
| SOFT_TTL | TX_SAD_1_LOW[31:16] | PN_THR (bit 3) | Early warning at ~75% PN space. Frame is normally sent. KaY should start negotiating new SAK. |
| HARD_TTL | TX_SAD_1_LOW[15:0] | PN_EXHAUST (bit 2) | Frame CRC is poisoned. SA can no longer be used. Set to 0 to trigger on PN overflow. |

Current SOFT_TTL = 0xC0000000 (75% of 2^32). Found bug: HARD_TTL was set to 0xFFFF (fires at 0xFFFF_0000, 65535 PNs before real overflow). Should be 0x0000 per databook to fire on actual PN overflow.

## 2.2 PN Exhaustion Netlink Notification

Implemented a netlink multicast notification path from driver to userspace MKA daemon:

Flow: Hardware IRQ -> dwc_handle_tx_sa_irq() -> osd_ops.macsec_pn_event() -> macsec_pn_event_handler() -> genlmsg_multicast("events" group)

Event attributes: ifname, controller (TX/RX), SA index, SC index, event type (threshold/exhausted).

Userspace MKA daemon receives event and performs SA rotation: disable old SA, create new SA with fresh key, enable new SA. The driver provides mechanism (SA CRUD via netlink), userspace provides policy (MKA protocol).

## 2.3 XPN and SALT

XPN (Extended Packet Numbering) is DISABLED (xpnsel=0). Without XPN, PN is 32-bit and SALT SRAM (for SSCI storage) is hardware-gated. At 10Gbps line rate, 32-bit PN exhausts in ~30 minutes of continuous full-rate traffic. Key rotation must be driven by userspace MKA protocol before PN exhausts.

# 3. SAI/SAD Table Indexing

## 3.1 Index Calculation

| Table | Dir | ENTRY Width | Total Entries | Index Range |
| --- | --- | --- | --- | --- |
| SAI | TX | 4 bits | 16 (per SC) | SC index: 0-15 |
| SAI | RX | 6 bits | 64 (per SA) | SA index: 0-63 |
| SAD | TX | 5 bits | 32 (per SA) | SA index: 0-31 |
| SAD | RX | 6 bits | 64 (per SA) | SA index: 0-63 |
| ARW | RX | 6 bits | 64 | SA index: 0-63 |

SA index = SC_index * SA_PER_SC + AN. With SA_PER_SC=2 and sequential SC allocation, SA indices are always contiguous 0,1,2,...63.

TX datapath: SAI match -> SC_index -> concatenate ACT_SA -> SAD[SC_index*2 + ACT_SA]. SAI uses SC-level indexing (16 entries), SAD uses SA-level indexing (32 entries).

RX datapath: SAI match SCI+AN -> SAI entry N -> same N -> SAD[N], ARW[N]. Both SAI and SAD use SA-level indexing (64 entries). Per databook: 'same entry number is taken for SAD, SALT and Anti-Replay Window.'

## 3.2 Fixed Issues

1. omni_macsec_get_sc_index: Replaced division-by-SA_PER_SC (which was dead code for DWC and buggy for old Omni) with direct software cache lookup returning -ENOENT.

2. omni_macsec_get_sc_index: Was hardcoding DWC_MACSEC_TX_SA_PER_SC for both TX and RX paths. Fixed to select divisor per ctlr (before the entire fallback was removed).

3. dwc_get_sc_lut_key_index: Returns base SA index (AN0). All callers correctly add curr_an or divide to get SC index. Function name is misleading but semantics are consistent.

# 4. Key Programming Paths

## 4.1 MACSEC_KEY_PROGRAM vs CONFIG_MACSEC

Both paths converge to the same hardware programming function (dwc_add_update_sa -> dwc_aes_key_program).

| Operation | MACSEC_KEY_PROGRAM | CONFIG_MACSEC |
| --- | --- | --- |
| Key source | Userspace genl netlink | Kernel macsec_context.sa.key |
| SAK to HW | dwc_aes_key_program() | Same function |
| SAD write | dwc_tx_sad_write() | Same function |
| SAI write | dwc_tx_sai_write() | Same function |
| H-key | SW computed, NOT written to DWC HW | Same (DWC derives H internally) |

MACSEC_KEY_PROGRAM is always defined in the current build. The #ifndef MACSEC_KEY_PROGRAM (TZ path) is dead code. When MACSEC_KEY_PROGRAM is disabled in the future (OP-TEE integration), the TZ path will activate to program keys through TrustZone.

## 4.2 H-Key (Hash Key)

DWC_macsec AES-GCM engine automatically derives H = AES_encrypt(K, 0^128) internally when the cipher key (SAK) is programmed. The databook key programming procedure (section 2.9.1) only mentions writing 'Key Value' into KEY_x registers - no separate H-key step exists.

The driver's hkey_generation() (AES encrypt of zero block via Linux crypto API) is for the old Omni LUT implementation. In the DWC path, hkey is stored in sc_info.hkey software cache but never written to DWC hardware registers. This is correct.

## 4.3 dwc_kt_config Read Path

KEY_x registers are hardware write-only (Access = W in databook). Added dwc_kt_read_cached() function that returns SAK and H-key from the macsec_lut_status software cache. On read request (rw != WRITE), the function derives SC index and AN from the key index, then looks up the cached key data.

# 5. Bypass Mechanism

## 5.1 Two Bypass Levels

Level 1 - Static Bypass (tx_bypass/rx_bypass in IP_CONTROL): When set, ALL frames bypass MACsec processing regardless of ethertype or SAI match. Used during init and for debug.

Level 2 - Ethertype Filter + VALIDATE_FRAMES (only for non-MACSEC frames, ethertype != 0x88E5):

- ethertype_filter_en + ETHERTYPE_FILTER[x] match -> bypass (EAPoL 0x888E, Slow Protocols 0x8809, LLDP 0x88CC)

- No ethertype match + VALIDATE_FRAMES=NOT_STRICT -> bypass (default during init)

- No ethertype match + VALIDATE_FRAMES=STRICT -> drop

Init state: NOT_STRICT. All non-MACSEC frames pass through. After SAs are configured, VALIDATE_FRAMES should be set to STRICT to enforce MACsec protection.

## 5.2 Broadcast and SCB

Broadcast/组播 data frames: Encrypted via MACsec SA (no bypass needed). MACsec CA provides security boundary - only stations with SAK can decrypt.

SCB (Single Copy Broadcast): TCI bit 2. When set in an SA's TCI, marks that SA as a broadcast SA. Currently SCB=0 in DWC_TCI_ENCRYPT_AUTH. For unicast-only MACsec this is correct. SCB support would require a dedicated SCB SA with SCB=1 in its TCI.

ARP broadcast: Source MAC matches SAI -> encrypted as normal MACsec frame -> sent. Receiver decrypts -> original ARP broadcast restored. ARP does NOT need bypass - it is protected by MACsec encryption, which also prevents ARP spoofing from outside the CA.

Only L2 control protocols at the physical link level must bypass: EAPoL (key agreement), PAUSE (flow control), LLDP (neighbor discovery). These operate link-local and cannot be encrypted.

## 5.3 dwc_set_bypass_entries

Programs ETHERTYPE_FILTER[x] registers during init:

| Slot | Ethertype | Protocol | Reason |
| --- | --- | --- | --- |
| Filt[0] | 0x888E | EAPoL (MKA) | Key agreement before MACsec established |
| Filt[1] | 0x8809 | Slow Protocols (PAUSE/LACP/OAM) | Flow control must not be encrypted |
| Filt[2] | 0x88CC | LLDP | Link discovery operates link-local |

# 6. VLAN Support

## 6.1 S-TAG and C-TAG (Q-in-Q)

S-TAG (Service TAG, TPID 0x88A8): Outer VLAN tag added by service provider for backbone forwarding.

C-TAG (Customer TAG, TPID 0x8100): Inner VLAN tag added by customer for internal VLAN segmentation.

Q-in-Q allows two customers to use the same C-VID without conflict because the outer S-VID distinguishes them at the provider edge.

## 6.2 VLANMODE Configuration

DWC_macsec has 7 VLAN modes configured via MACsec_IP_Control.vlanmode[31:29]. Current implementation sets vlanmode=000 (Not on the clear):

In vlanmode=000, ALL VLAN tags are encrypted and authenticated together with the payload. The VLAN information is invisible to intermediate switches. This is the safest default for direct-attach links.

The VLANMODE is explicitly configured in dwc_macsec_initialize() with:

val &= ~DWC_MACSEC_CTRL_VLANMODE_MASK;

val |= (DWC_MACSEC_VLANMODE_NOT_ON_CLEAR << DWC_MACSEC_CTRL_VLANMODE_SHIFT);

Fixed register definition bug: DWC_MACSEC_CTRL_VLANMODE_SHIFT was 3 (incorrect for bits 31:29), corrected to 29.

# 7. Driver Improvements Summary

| Change | Description |
| --- | --- |
| PN event notification | Added macsec_pn_event OSD callback + genl multicast "events" group for PN threshold/exhaustion events from ISR to userspace. |
| omni_macsec_get_sc_index | Removed dead fallback path (osi_macsec_get_sc_lut_key_index + division). Direct software cache lookup only. |
| dwc_kt_config read | Added dwc_kt_read_cached() to return SAK+H-key from software cache on read requests. |
| Ethertype bypass filters | Added 0x8809 (Slow Protocols) and 0x88CC (LLDP) to bypass filters alongside existing 0x888E (EAPoL). |
| VLANMODE_SHIFT fix | Fixed from 3 to 29 to match IP_CONTROL bits 31:29. |
| vlanmode=000 explicit | Added explicit VLANMODE=000 configuration in dwc_macsec_initialize(). |
| HARD_TTL default | Should be changed from 0xFFFF to 0x0000 per databook (0 = trigger on PN overflow). Not yet fixed. |
| macsec_enable gap | dwc_macsec_enable(OSI_ENABLE) is never called. MACsec engine may stay in bypass. Not yet fixed. |