DWC_macsec Driver

Design Analysis & Implementation Notes

DWC_macsec 1.05a | omnieth Driver | Linux Kernel

Generated: May 2026

# 1. DWC_macsec Memory Architecture

The DWC_macsec 1.05a IP configuration specifies distinct memory types for each internal table. SAI (Security Association Identifier) uses internal flip-flops for low-latency lookup, while all other data tables use external SRAM.

## 1.1 Memory Type Summary

| Module | Description | Entries | Memory Type | Status |
| --- | --- | --- | --- | --- |
| TX SAI | TX lookup table (per-SC) | 16 | Internal FFs | Active |
| RX SAI | RX lookup table (per-SA) | 64 | Internal FFs | Active |
| TX SAD | TX SA parameters (PN, TCI, key) | 32 | External Dual Port SRAM | Active |
| RX SAD | RX SA parameters (PN, replay) | 64 | External Dual Port SRAM | Active |
| TX SALT | XPN SSCI storage | 32 | External Two Port SRAM | Inactive (XPN=0) |
| RX SALT | XPN SSCI storage | 64 | External Two Port SRAM | Inactive (XPN=0) |
| RX ARW | Anti-Replay Window bitmap | 64 | External Two Port SRAM | Active |
| TX MIB SA | Per-TX-SA MIB counters | 64 | External Two Port SRAM | Active |
| RX MIB SA | Per-RX-SA MIB counters | 320 | External Two Port SRAM | Active |
| RX MIB SC | Per-RX-SC MIB counters | 320 | External Two Port SRAM | Active |
| AES Cipher Key | AES-GCM cipher key (SAK) | 98 | External SRAM (297-bit) | Active |
| AES Hash Key | AES-GCM hash key (H) | 98 | External SRAM (294-bit) | Active |
| AES Cipher Ctx | AES-GCM cipher context | 98 | External SRAM (150-bit) | Active |

## 1.2 Design Rationale

SAI uses Internal FFs because the lookup is on the critical datapath and requires minimal latency (direct decoding, 1-2 pipeline stages). With only 16 TX + 64 RX entries at ~100 bits each, the flip-flop area is acceptable (~4.8K FFs).

SAD/SALT/MIB/ARW use external SRAM because each entry is significantly larger (SAD: 200+ bits per entry, MIB: 39-78 bits) and requires true read-modify-write capability (PN counter increments on every packet). Dual-port SRAM supports simultaneous read and write for datapath throughput.

# 2. SAI vs SAD Design

## 2.1 Two-Table Architecture

SAI (Security Association Identifier) and SAD (Security Association Data) serve fundamentally different purposes in the MACsec pipeline:

|  | SAI (Lookup Table) | SAD (Parameter Table) |
| --- | --- | --- |
| Purpose | Which SA does this frame belong to? | What are this SA's parameters? |
| Input | Frame fields (DA/SA/VLAN/SCI/AN) | SA index (output from SAI lookup) |
| Output | SA index + outcome | PN, key index, TCI, MTU, TTL |
| Write freq. | Only during SA configuration | Every packet (PN update) |
| Memory | Internal FFs (low latency) | External Dual Port SRAM |
| TX entries | 16 (one per SC) | 32 (SC x2 AN per SC) |
| RX entries | 64 (one per SA) | 64 (same index as SAI) |

## 2.2 TX Datapath: Two-Level Indirection

On TX, the SAI maps a frame to an SC index. The hardware then concatenates SC_index with the active AN bit (ACT_SA) to form the SAD index:

Frame -> SAI[SC_index] -> SC_index + ACT_SA -> SAD[SC*2+AN]

## 2.3 RX Datapath: Direct Mapping

On RX, the SAI entry number IS the SAD entry number. The databook states: 'same entry number is taken for the SAD, SALT and Anti-Replay Window.'

Frame -> SAI[N] (match SCI+AN) -> SAD[N] -> decrypt

This means each RX SAI entry corresponds to exactly one SA (not one SC). With 32 SCs x 2 ANs = 64 SAI entries and 64 SAD entries.

# 3. TCI and ARW

## 3.1 TCI (TLS/Token Control Information)

TCI is stored in TX SAD (TX_SAD_0_HIGH bits [6:2]) and is an OUTPUT parameter written into the outgoing frame's SecTAG, not a matching rule for SAI lookup.

| Bit | Name | Meaning |
| --- | --- | --- |
| 4 | ES | End Station: source/destination are both end stations |
| 3 | SC | Secure Channel: supports SC behavior |
| 2 | SCB | Single Copy Broadcast: broadcast via shared SAK |
| 1 | E | Encryption: payload is encrypted |
| 0 | C | Changed text: integrity protection is applied |

Hardware reads TCI from SAD when encrypting a TX frame and inserts these bits into the SecTAG header. The receiver uses them to determine what security processing was applied.

## 3.2 ARW (Anti-Replay Window)

ARW is a per-RX-SA table (64 entries x 11-bit, External Two Port SRAM) that stores the replay window state for duplicate/replay detection.

### Working Principle

Each RX SA maintains a sliding PN window:

Past <- ARW window -> Present (PN_high = highest valid PN received)

Incoming frame PN:

PN < PN_low     -> drop (too old, possible replay)

PN in window    -> check bitmap: already received? drop : accept

PN > PN_high    -> accept, slide window forward

ARW is configured in Logarithmic mode (DWC_MACSEC_WIN_SIZE_MODE = Logarithmic), where the window size is a power-of-2 exponent, significantly reducing hardware resource consumption.

# 4. SOFT_TTL vs HARD_TTL

## 4.1 Two-Level PN Warning Mechanism

Each TX SA has two threshold fields in TX_SAD_1_LOW, providing a two-stage early warning system before PN (Packet Number) exhaustion:

| Threshold | Register | IRQ Bit | Hardware Behavior |
| --- | --- | --- | --- |
| SOFT_TTL | TX_SAD_1_LOW[31:16] | SOFT_TTL_STAT (bit 3) | IRQ only. Frame transmitted normally. KaY should start SAK negotiation. |
| HARD_TTL | TX_SAD_1_LOW[15:0] | HARD_TTL_STAT (bit 2) | IRQ + frame CRC poisoned. SA forcibly stopped. |
| PN Overflow | Auto (32-bit wrap) | N/A | SA becomes invalid. PN=0 detected if PNSE_CHECK_EN=1. |

Both SOFT_TTL and HARD_TTL store the upper 16 bits of the threshold value; the lower 16 bits are implied as zero. The effective trigger PN is: threshold = FIELD << 16.

## 4.2 Current Code Values

DWC_DEFAULT_PN_THRESHOLD = 0xC0000000U         // SOFT_TTL: ~75% PN space

soft_ttl_hi = (0xC0000000 >> 16) = 0xC000      // trigger at PN >= 0xC000_0000

hard_ttl_hi = 0xFFFFU                          // trigger at PN >= 0xFFFF_0000

## 4.3 HARD_TTL Bug

Per the DWC_macsec databook: 'Generally, this field is set to zero which will cause a HARD_TTL event to occur when the PN number overflows.'

The current driver sets hard_ttl_hi = 0xFFFF, which triggers HARD_TTL at PN >= 0xFFFF_0000 (99.998% of PN space). This wastes 65,535 valid PN values and fires before actual overflow.

Fix: change hard_ttl_hi to 0x0000 to let hardware detect actual PN overflow.

Key rotation (when PN exhausts) is handled by the userspace MKA (MACsec Key Agreement) daemon. The driver only reports the event and provides create/delete/enable SA mechanisms via netlink.

# 5. XPN and SALT

XPN (Extended Packet Numbering) extends the PN from 32-bit to 64-bit by adding a 32-bit SSCI (Short SCI). SALT stores the SSCI per SA.

Current configuration: DWC_MACSEC_XPN_SEL = 0 (XPN disabled). SALT SRAM exists physically (TX: 32x111bit, RX: 64x111bit) but is hardware-gated and inactive.

|  | XPN=0 (Current) | XPN=1 |
| --- | --- | --- |
| PN width | 32-bit (2^32 ~ 4.3B pkts) | 64-bit (2^64, effectively infinite) |
| PN exhaust @10Gbps | ~30 minutes | Never |
| SALT SRAM | Inactive (no power/latency) | Active (extra SRAM read per SA) |
| Key hierarchy | SAK directly | SAK -> SSCI + PN |
| Use case | Enterprise, Automotive | Carrier/Data Center high-speed |

At 25Gbps line rate, 32-bit PN exhaustion occurs in approximately 12 minutes. This is manageable with proper SOFT_TTL early warning and key rotation.

# 6. PN Exhaustion Notification Gap

## 6.1 Problem

The driver's IRQ handler (dwc_handle_tx_sa_irq) detected PN threshold/exhaustion interrupts but only incremented statistics counters and logged messages. No notification was sent to userspace, leaving the MKA daemon unaware of the pending PN exhaustion.

## 6.2 Solution

Added a callback mechanism from the OSI layer to the OSD layer, which sends a genl multicast notification to userspace.

## 6.3 Data Flow

Hardware PN threshold/exhaustion IRQ

-> dwc_handle_tx_sa_irq(sa_idx)

-> osi_core->osd_ops.macsec_pn_event(osd, sa_idx, sc_idx, event, ctlr)

-> macsec_pn_event_handler() in macsec.c

-> genlmsg_multicast("events" mc group)

-> Userspace MKA daemon receives notification

-> Key rotation: create new SA, switch AN, delete old SA

## 6.4 Files Modified

| File | Change |
| --- | --- |
| hardware/include/osi_core.h | Added macsec_pn_event callback to osd_core_ops + OSI_MACSEC_PN_EVENT_* macros |
| hardware/osi/macsec/dwc_macsec.c | Call callback from dwc_handle_tx_sa_irq on PN_THR / PN_EXHAUST |
| ether_export.h | Added OB_MACSEC_CMD_PN_EVENT + ob_macsec_pn_event_attrs enum |
| macsec.h | Declared macsec_pn_event_handler() prototype |
| macsec.c | Implemented genl multicast notification + registered 'events' mc group |
| osd.c | Assigned macsec_pn_event callback in ether_assign_osd_ops() |

# 7. ip macsec Key Rotation Workflow

The driver supports the standard Linux MACsec offload interface (macsec_ops). Userspace uses ip macsec commands for testing and the MKA daemon for production.

## 7.1 Key Rotation Commands

# Initial SA setup (AN=0, key_id=00)

ip macsec add eth0 tx sa 0 pn 1 on key 00 <SAK_hex>

ip macsec add eth0 rx sci 0102030405060708 sa 0 pn 1 on key 00 <SAK_hex>

# ---- PN threshold reached, MKA negotiates new SAK ----

# Create new TX SA (AN=1, key_id=01, fresh PN=1)

ip macsec add eth0 tx sa 1 pn 1 on key 01 <new_SAK_hex>

ip macsec add eth0 rx sci 0102030405060708 sa 1 pn 1 on key 01 <new_SAK_hex>

# Switch active SA to new AN

ip macsec set eth0 tx sa 1 active on

# ---- Old SA no longer used ----

ip macsec del eth0 tx sa 0

ip macsec del eth0 rx sci 0102030405060708 sa 0

## 7.2 AN (Association Number)

AN is a 2-bit field in the SecTAG (0-3), meaning each SC supports up to 4 SAs. Key rotation typically alternates between AN 0 and AN 1. The driver currently supports AN 0 and AN 1 per the DWC_macsec 2-SA-per-SC configuration.

# 8. SAI/SAD Table Index Analysis

## 8.1 Index Space Summary

| Table | Direction | ENTRY Width | Total Entries | Index Range |
| --- | --- | --- | --- | --- |
| SAI | TX | 4 bits (LOG2_NUM_SC) | 16 | 0-15 (SC index) |
| SAI | RX | 6 bits (LOG2_NUM_SA) | 64 | 0-63 (SA index = SC*2+AN) |
| SAD | TX | 5 bits (LOG2_NUM_SA) | 32 | 0-31 (SA index = SC*2+AN) |
| SAD | RX | 6 bits (LOG2_NUM_SA) | 64 | 0-63 (SA index = SC*2+AN) |
| ARW | RX | 6 bits | 64 | 0-63 (same as SAD) |
| SC_CORR | RX | 6 bits | 64 | 0-63 (SA->SC mapping) |

## 8.2 TX: Two-Level Indirection

SAI entry stores SC_index (0-15). Hardware concatenates SC_index with ACT_SA bit to form SAD index:

SAI[SC_index] -> SC_index + ACT_SA -> SAD[SC_index*2 + AN]

SAI = 16 entries (per-SC), SAD = 32 entries (per-SA)

## 8.3 RX: Direct One-to-One Mapping

SAI entry N directly maps to SAD[N], ARW[N]. No indirection.

SAI[N] (match SCI+AN) -> SAD[N] -> decrypt

SAI = SAD = 64 entries (per-SA)

## 8.4 SA Index Continuity

SC indices are allocated sequentially from 0 by dwc_get_avail_sc_idx(). SA index = SC * SA_PER_SC + AN, which produces a continuous 0, 1, 2, 3, ..., 63 sequence with no gaps. Even if a SC is deleted and re-created at a different index, the SA index space is direct-mapped and does not require contiguity.

# 9. omni_macsec_get_sc_index Fix

## 9.1 Issue

The function searched macsec_lut_status[] (software cache) first. If not found, it fell back to osi_macsec_get_sc_lut_key_index() which returns key_index = sc_idx * SA_PER_SC, then divided by SA_PER_SC to recover sc_idx. This fallback was dead code for DWC because dwc_get_sc_lut_key_index searches the same macsec_lut_status[] array.

Additionally, the division used DWC_MACSEC_TX_SA_PER_SC regardless of controller direction, which was only correct because both TX and RX happen to have SA_PER_SC = 2.

## 9.2 Fix Applied

- Removed the fallback path (osi_macsec_get_sc_lut_key_index + division by SA_PER_SC)

- Replaced with return -ENOENT when SCI not found in software cache

- Removed unused key_index and ret variables

# 10. Key Programming: MACSEC_KEY_PROGRAM vs CONFIG_MACSEC

## 10.1 Path Convergence

Both key programming paths ultimately converge to the same hardware write function:

dwc_add_update_sa() -> dwc_aes_key_program(sa_idx, sak, key_len, encrypt)

|  | MACSEC_KEY_PROGRAM | CONFIG_MACSEC |
| --- | --- | --- |
| Key source | genl userspace (tx_sa.sak) | kernel macsec_context.sa.key |
| Entry point | macsec_create_tx_sa() | omni_macsec_add_txsa() |
| SAK write | dwc_aes_key_program() | Same function |
| SAD write | dwc_tx_sad_write() | Same function |
| SAI write | dwc_tx_sai_write() | Same function |
| H-key | SW computed, not written to HW | Same behavior |
| TZ path | #ifndef MACSEC_KEY_PROGRAM (dead) | Same (dead) |

## 10.2 Build Configuration

In the current build, MACSEC_KEY_PROGRAM is always defined (-DMACSEC_KEY_PROGRAM in Makefile). This means:

- Key is written directly by driver via dwc_aes_key_program()

- #ifndef MACSEC_KEY_PROGRAM TrustZone path is dead code

- H-key is computed in software (hkey_generation) but DWC hardware derives H internally from SAK, so the SW H-key is never written to AES registers

## 10.3 Future OP-TEE Migration

When keys move to OP-TEE secure domain:

- Remove -DMACSEC_KEY_PROGRAM from Makefile

- Activate #ifndef MACSEC_KEY_PROGRAM TZ path

- osi_macsec_config() writes only SAI/SAD (no key)

- TZ path: macsec_tz_kt_config() -> OP-TEE -> secure key write

- CONFIG_MACSEC: kernel ctx->sa.key becomes empty/dummy; real key in OP-TEE

# 11. dwc_kt_config Read Logic

The DWC_macsec KEY_x registers (offsets 0x20-0x3C) are documented as 'W' (write-only) in the databook. Hardware key readback via APB registers is not possible.

## 11.1 Before Fix

dwc_kt_config() returned -1 for any read operation with the error message 'AES key read not supported'. This was functionally correct (HW does not support reads) but prevented any caller from retrieving cached key material.

## 11.2 After Fix

Added dwc_kt_read_cached() which resolves the SA index to SC index and AN, then looks up the cached SAK from macsec_lut_status[ctlr].sc_info[].sak[].

dwc_kt_read_cached(index=SA_idx, ctlr, &kt_config):

sc_idx = index / SA_PER_SC

an = index % SA_PER_SC

-> search macsec_lut_status[ctlr].sc_info[]

-> match sc_idx_start && an_valid

-> memcpy sak[32] + hkey[16] (if MACSEC_KEY_PROGRAM)

-> set flags |= ENTRY_VALID

This provides a software-cached fallback for key reads that is always consistent with the last written key.

# 12. Bypass Mechanism

## 12.1 Two-Layer Bypass Architecture

Frame arrives at MACsec engine

|

+-- tx_bypass=1 / rx_bypass=1 ?

|     YES -> ALL frames bypassed (global override)

|     NO  -> continue

|

+-- Ethertype == 0x88E5 (MACsec) ?

|     YES -> SAI lookup -> SAD -> encrypt/decrypt

|     NO  -> non-MACsec frame

|

+-- Non-MACsec frame:

ethertype_filter_en=1 && ethertype in FILTER[x]?

YES -> bypass

NO  -> VALIDATE_FRAMES mode:

non-strict -> bypass

strict     -> drop

## 12.2 Ethertype Bypass Filters Configured

| Slot | Ethertype | Protocol | Rationale |
| --- | --- | --- | --- |
| Filt[0] | 0x888E | EAPoL (MKA) | Key agreement must work before MACsec is established |
| Filt[1] | 0x8809 | Slow Protocols | PAUSE/LACP/OAM/MRP - L2 control, must not be encrypted |
| Filt[2] | 0x88CC | LLDP | Link discovery, single-hop neighbor protocol |

## 12.3 Initialization Flow

dwc_macsec_initialize() sets:

- tx_bypass = 1, rx_bypass = 1 (all traffic bypassed during init)

- MIB_VALIDATE = NOT_STRICT (non-MACsec frames not dropped)

- Ethertype bypass filters programmed (EAPoL, Slow Proto, LLDP)

## 12.4 Discovered Gap

dwc_macsec_enable(OSI_ENABLE) is NEVER called. After initialization, the MACsec engine stays in global bypass mode (tx_bypass=1, rx_bypass=1). Even after SAs are configured, all traffic continues to bypass MACsec processing.

Fix needed: call dwc_macsec_enable(OSI_ENABLE) when the first SA is successfully created and enabled, clearing the tx_bypass/rx_bypass bits to activate MACsec processing on the datapath.

# 13. Broadcast/Multicast/ARP Bypass Decision

## 13.1 CA (Connectivity Association) Boundary

MACsec encrypts traffic within a CA - a set of stations sharing the same SAK. The encryption scope is defined by the CA boundary, not by the packet's destination MAC address type.

CA Internal (shared SAK)          CA External

+------------------------+       +--------+

| Station A <-> Station B |       |Attacker|

| Broadcast: A -> all     |   X   |No SAK  |

| ARP: who has IP?       |       |Decrypt |

| All MACsec encrypted    |       | fails  |

+------------------------+       +--------+

## 13.2 Decision Rule

|  | Must Bypass | Go Through MACsec |
| --- | --- | --- |
| Scope | Link-local (single physical hop) | Network-wide (multi-hop) |
| Examples | EAPoL, PAUSE, LACP, LLDP, OAM | ARP, IP broadcast, IP multicast |
| Reason | Link-layer control; encrypting breaks L2 functions | Protected by CA boundary; external attackers cannot decrypt |
| Key property | Must work before/without SAK | SAK shared within CA |

Broadcast, multicast, and ARP frames go through normal MACsec SAI lookup and encryption using the SCB (Single Copy Broadcast) TCI mode. The sending station encrypts once; all CA members decrypt with the shared SAK. External attackers without the SAK cannot read or forge these frames.

End of Document